RADIO ATLAS / SOLARASPHERE

# Reachability vs. Detectability

Backlog Addition Report & Project Roadmap

Story 6.17

Distinguish geometric HF path reachability from expected receiver detectability

Prepared for Radio Atlas / SolaraSphere
October 2026

# 1. Executive Summary

Recommendation. Add Story 6.17 as a bounded extension of the existing PyRayHF + PSK Reporter propagation experiment. The feature should explicitly model two different questions: (1) does the ionosphere support a plausible ray path, and (2) would the resulting signal likely exceed the effective receiver detection threshold after propagation loss, absorption, noise, antenna/system effects, and modulation requirements are considered?

Keeping these predictions separate prevents a scientifically important category error. A successful ray trace is evidence of geometric support, not proof that a receiver can decode the transmission. Likewise, a missing PSK Reporter spot is not automatically evidence of propagation failure.

The recommended implementation is incremental: preserve the current binary reachability MVP, introduce nullable detectability fields and model interfaces, then add field-strength, absorption, and noise/reliability components in independently testable stages. This lets the experiment mature without forcing PyRayHF to become a full link-budget engine.

| Primary output | A prediction record that distinguishes ionospheric reachability, signal-level estimate, and receiver detectability. |
| --- | --- |
| Scientific benefit | Residuals become interpretable: ray-path error, loss/absorption error, or receiver/noise uncertainty can be analyzed separately. |
| Architecture benefit | Propagation engines, loss models, noise models, and receiver profiles remain swappable behind interfaces. |
| MVP boundary | Do not require receiver-specific thresholds or full absorption modeling to ship the first version; fields remain nullable until evidence supports them. |

# 2. Research Rationale

Recent work strongly supports treating HF path existence and practical reception as related but distinct quantities:

PyRayHF establishes the ray-geometry layer. Forsythe et al. validate fast HF ray-tracing forward operators against a 3-D reference tracer, making it a strong foundation for geometric reachability and path observables.

Received field strength needs its own model layer. Zhu et al. compare REC533, ICEPAC, and VOACAP against measured HF field-strength data and show material differences in bias and dispersion across propagation regimes.

Absorption can suppress an otherwise supported path. Verhulst et al. show that passive HF observations can detect enhanced absorption caused by solar X-rays and particle precipitation.

Noise sets the practical floor. ITU-R P.372 models atmospheric, galactic, and man-made radio noise, which directly affects whether a received signal is usable.

Reliability is a probabilistic system outcome. ITU-R P.842 combines predicted receiver signal power, noise, variability, and required SNR into HF reliability calculations.

# 3. Target Conceptual Model

Ionosphere state
      |
      v
  PyRayHF / ray model
      |
      +--> predicted_reachable
      |
      v
Field-strength / path-loss model
      |
      +--> predicted_field_strength / path_loss
      |
      v
Absorption model
      |
      +--> estimated_absorption
      |
      v
External + receiver noise model
      |
      v
Receiver / mode threshold model
      |
      +--> predicted_detectable + confidence
      |
      v
PSK Reporter observation
      |
      v
Residual + error taxonomy

Design rule: No downstream layer should overwrite the upstream physical conclusion. A path may be geometrically reachable but predicted undetectable, and that distinction must remain visible in persisted data and evaluation metrics.

# 4. Proposed Domain and Schema Changes

PropagationPrediction
  predicted_reachable: bool | None
  predicted_detectable: bool | None

  predicted_field_strength_dbuv_m: float | None
  predicted_receiver_power_dbm: float | None
  estimated_path_loss_db: float | None
  estimated_absorption_db: float | None
  estimated_noise_floor_dbm: float | None
  required_snr_db: float | None
  receiver_threshold_dbm: float | None

  reachability_model: str
  signal_model: str | None
  noise_model: str | None
  receiver_profile_id: str | None

  reachability_confidence: float | None
  detectability_confidence: float | None
  model_version: str

## Observation status

observation_status:
  OBSERVED
  CREDIBLE_NEGATIVE
  UNKNOWN_RECEIVER_STATE
  UNKNOWN_TRANSMITTER_STATE
  OUT_OF_MONITORED_BAND
  INSUFFICIENT_METADATA

The evaluation layer should avoid coercing unknown observations into false negatives. Binary confusion-matrix metrics should only use observations that meet the experiment’s credibility rules.

# 5. Project Roadmap

| Phase | Purpose | Primary deliverable | Story |
| --- | --- | --- | --- |
| Phase 0 - Contract and baseline | Define terminology, units, interfaces, nullability rules, and experiment versioning. Freeze the current reachability-only output as a regression baseline. | Schema + ADR + regression fixture | Story 6.17A |
| Phase 1 - Reachability layer | Keep PyRayHF focused on ray support, path geometry, hop count, apex, elevation/takeoff angle, and solver metadata. | Reachability predictor contract | Story 6.17B |
| Phase 2 - Field-strength benchmark | Add a model-independent signal prediction interface. Implement or wrap one field-strength baseline, preferably ITU-R P.533/REC533, while retaining VOACAP as a comparison candidate. | Signal-level predictor + benchmark | Story 6.17C |
| Phase 3 - Absorption context | Add an absorption component that can first ingest externally derived/event-based absorption indicators, then later graduate to a quantitative model. | Absorption provider contract | Story 6.17D |
| Phase 4 - Noise floor | Implement an external-noise provider using ITU-R P.372-compatible outputs. Keep location, frequency, time, and environment assumptions explicit. | Noise provider | Story 6.17E |
| Phase 5 - Detectability decision | Combine receiver power, noise, required SNR, and receiver/mode profile into a probabilistic or threshold-based detectability decision. | Detectability evaluator | Story 6.17F |
| Phase 6 - Evaluation and residual taxonomy | Classify disagreements by layer: geometry miss, field-strength miss, absorption/noise miss, or observation uncertainty. Add stratified metrics. | Residual taxonomy + metrics | Story 6.17G |
| Phase 7 - Visualization | Add timeline and map views that independently display reachable, detectable, observed, and unknown states. | Evaluation UI | Story 6.17H |

# 6. Acceptance Criteria by Milestone

M1 - Reachability contract: Given TX/RX coordinates, timestamp, and frequency, return a versioned ray-path prediction without requiring receiver metadata.

M2 - Signal strength: For the same experiment record, compute a separately versioned field-strength or receiver-power estimate using a documented model and units.

M3 - Noise and threshold: Represent external noise and receiver/mode threshold independently; missing receiver data yields unknown detectability rather than false.

M4 - Detectability: Return predicted_detectable only when the minimum evidence contract is met; otherwise return null/unknown with a reason code.

M5 - Evaluation: Compare predictions with PSK Reporter observations without treating missing reports as negatives unless activity rules support a credible negative.

M6 - Reproducibility: Persist all model names, versions, inputs, assumptions, and units required to replay an experiment run.

# 7. Evaluation Plan

## 7.1 Unit and contract tests

Unit conversions are explicit and tested (dBµV/m, dBm, dB, Hz/MHz).

A reachable path may be undetectable without violating validation rules.

Unknown receiver threshold produces predicted_detectable = null, not false.

Each provider records model name/version and input assumptions.

A replay of identical inputs produces the same deterministic baseline result where the model itself is deterministic.

## 7.2 Scientific evaluation

Stratify residuals by frequency/band, distance, path azimuth, day/night, season, solar activity, and geomagnetic disturbance.

Report reachability metrics separately from detectability metrics.

Measure calibration if detectability becomes probabilistic (reliability curve / Brier score), not only accuracy.

Maintain an UNKNOWN bucket and report its rate; a shrinking unknown rate is a data-quality metric, not merely a nuisance.

Benchmark at least one classical received-field-strength model before training a learned correction model.

# 8. Major Risks and Guardrails

| Risk | Why it matters | Guardrail |
| --- | --- | --- |
| Receiver metadata scarcity | PSK Reporter generally does not provide complete antenna gain, receiver NF, local noise environment, or decoder threshold. | Treat receiver profiles as optional and confidence-scored; do not fabricate a station-specific threshold. |
| Observation bias | Reporting stations are not uniformly distributed and may change band/mode availability over time. | Maintain receiver-activity inference and cohort-based evaluation. |
| Model mismatch | P.533/VOACAP-like models predict statistical/median behavior, while an individual FT8 observation is instantaneous. | Keep timescale semantics in metadata and compare like with like. |
| Noise uncertainty | Local man-made noise can dominate generic global noise models. | Expose assumed environment class and uncertainty instead of presenting a single precise threshold. |
| Feature creep | A full link budget, antenna database, decoder model, absorption model, and receiver calibration program can balloon the experiment. | Ship interfaces first, one baseline implementation per layer, and preserve nullability. |

# 9. Research Library Recommendations

Recommended policy: Add the first five items to the core Radio Atlas/SolaraSphere research library now. Add the signal-classification papers to the SolaraSignal branch. Keep the storm-composition and plasma-bubble papers as secondary context rather than core backlog references.

## 9.1 Core: add now

| Source | Identifier | Why add it | Priority |
| --- | --- | --- | --- |
| PyRayHF: An Open Source Ray-Tracing Forward Operator for Assimilating Oblique Ionograms Forsythe et al., Radio Science (2026) | 10.1029/2026RS008702 | Foundation for the reachability layer and model-interface design. | CORE |
| Exploring the Applicability of Three Popular Models for Predicting the Received Field Strength of HF Sky-Wave Propagation and Communication Zhu et al., Space Weather (2026) | 10.1029/2025SW004907 | Direct evidence for separating ray support from received field-strength prediction; strong benchmark-design reference. | CORE |
| Radio-Quiet Ionosonde Observations for Monitoring Absorption in the HF Band Verhulst et al., Radio Science (2026) | 10.1029/2026RS008688 | Supports an explicit absorption layer and shows passive HF signals can reveal absorption events. | CORE |
| Recommendation ITU-R P.533-14: Method for the prediction of the performance of HF circuits ITU-R | P.533-14 | Primary classical reference for HF frequencies, field strength, and circuit performance; candidate baseline implementation. | CORE |
| Recommendation ITU-R P.372-18: Radio noise ITU-R (2026) | P.372-18 | Defines external radio-noise modeling needed to convert received signal estimates into a usable SNR context. | CORE |
| Recommendation ITU-R P.842-2: Computation of reliability and compatibility of HF radio systems ITU-R | P.842-2 | Useful system-level bridge from signal power and noise to reliability; valuable for future probabilistic detectability. | CORE |

## 9.2 SolaraSphere context: useful, but not required for Story 6.17

Iono-PhyTAU: A Lightweight Global Ionospheric TEC Prediction Model Based on Temporal Attention Unit and Physical Condition Constraints of the Ionosphere. Useful for future physics-informed SolaraSphere forecasting and storm-period evaluation, but not needed to implement detectability. Source

Localized ΣO/N₂ Enhancements During Non-Superstorms: Tongue, Vortex, and the Tongue-to-Vortex Transition. Supports the case that storm-time composition is spatially structured; useful when interpreting regional residuals. Source

Equatorial Plasma Bubbles Modulate the Directional Coherence of the Ionosphere. Useful for azimuth- and latitude-stratified error analysis, especially at low latitudes. Source

## 9.3 SolaraSignal branch: add to the signal-classification library

BR-FiLM: Bounded Residual Channel-Quality Conditioning for Automatic Modulation Recognition. Strong reference for SNR-conditioned classification and low-SNR evaluation. Source

Sub-6 GHz Over-the-Air AMC via Curriculum Fine-Tuned CNN-Transformers. Strong methodological reference for validating classifiers on real RF links rather than only synthetic or cabled data. Source

# 10. Suggested Library Metadata

title
authors
year
venue
doi_or_standard_id
source_url
accessed_at
project_tags[]
  - radio-atlas
  - solarasphere
  - propagation-validation
  - reachability
  - field-strength
  - absorption
  - radio-noise
  - detectability
priority: core | supporting | solara-signal
implementation_notes
related_story_ids[]

For standards, store the recommendation number and revision explicitly. For papers, store DOI plus a stable publisher or arXiv URL. This will make later agent-assisted architecture work much less brittle than relying on filenames alone.

# 11. Backlog Addition: Ready-to-Paste Story

Story 6.17 - Distinguish Geometric Reachability from Signal Detectability

As a Radio Atlas researcher, I want the propagation experiment to distinguish whether an HF ray path is physically supported from whether the resulting signal is likely to be detectable by a receiving system, so that prediction residuals can be interpreted correctly and future models can be evaluated against real observations without conflating ray geometry, attenuation, noise, and receiver performance.

Acceptance criteria:

Reachability remains a first-class, independently persisted prediction.

Detectability is independently represented and may be unknown when signal/noise/receiver inputs are insufficient.

Field strength, path loss, absorption, external noise, receiver threshold, and required SNR are separately represented with explicit units.

Provider/model name and version are persisted for every computed layer.

Evaluation code does not treat missing PSK Reporter records as negatives unless the observation satisfies credible-negative rules.

Metrics can be reported separately for reachability and detectability.

# 12. Source Links

Forsythe et al. (2026), PyRayHF: https://agupubs.onlinelibrary.wiley.com/doi/10.1029/2026RS008702

Zhu et al. (2026), HF field-strength model comparison: https://agupubs.onlinelibrary.wiley.com/doi/10.1029/2025SW004907

Verhulst et al. (2026), HF absorption monitoring: https://agupubs.onlinelibrary.wiley.com/doi/10.1029/2026RS008688

ITU-R P.533-14: https://www.itu.int/dms_pubrec/itu-r/rec/p/R-REC-P.533-14-201908-I!!TOC-HTM-E.htm

ITU-R P.372-18: https://www.itu.int/dms_pubrec/itu-r/rec/p/R-REC-P.372-18-202609-I!!SUM-HTM-E.htm

ITU-R P.842-2: https://www.itu.int/dms_pubrec/itu-r/rec/p/R-REC-P.842-2-199907-S!!PDF-E.pdf

Iono-PhyTAU: https://agupubs.onlinelibrary.wiley.com/doi/10.1029/2026SW005211

Yu et al. (2026), ΣO/N₂ storm structures: https://agupubs.onlinelibrary.wiley.com/doi/10.1029/2026JA036029

Liu et al. (2026), equatorial plasma bubble coherence: https://agupubs.onlinelibrary.wiley.com/doi/10.1029/2026JA035960

BR-FiLM: https://arxiv.org/abs/2608.18395

Sub-6 GHz Over-the-Air AMC: https://arxiv.org/abs/2609.07726